10 Moulton Street

Cambridge, MA  02238

Telephone (617)491-1850

Telex No. 92-1470

Bolt Beranek and Newman Inc.                                              bbn

18 February 1983

G. Robert Blakey

Professor of Law

Notre Dame Law School

Notre Dame, Indiana  46556

Dear Bob,

This letter conveys our suggestions for further studies of the

acoustical evidence that we examined for the House Select Committee

on Assassinations.

We have read the report that Norman Ramsey's committee wrote

for the National Academy of Sciences (NAS).  That report can be

divided into two parts.  One part analyzes some data that pertain

to the Barber hypothesis, while the other part contains opinions

about our report.  Only within the former part do we find important

and original information.

Barber discovered a very weak spoken phrase on the DPD Dictabelt

recording that is heard at about the time of the sound impulses we

concluded were probably caused by the fourth shot.  The NAS Committee

has shown to our satisfaction that this phrase has the same origin

as the same phrase heard also on the Audograph recording.  The

Audograph recording was originally made from the Channel 2 radio.

The common phrase is heard on Channel 2 about a minute after the

assassination would appear, from context, to have taken place.

Therefore, it would seem, and the NAS Committee concludes, that the

sounds that we connected with gunfire were made about a minute after

the assassination shots were fired.

Upon reading the NAS report, we did a brief analysis of the

Audograph dub that was made by the NAS Committee and loaned to us

by them.  We found some enigmatic features of this recording that

occur at about the time that individuals react to the assassination.

Therefore, we have doubt about the time synchronization of events

on that recording, and so we doubt that the Barber hypothesis is

proven.  The NAS Committee did not examine the several items of

evidence that corroborated our original findings, so that we still

agree with the House Select Committee on Assassinations conclusion

that our findings were corroborated.

Boston   Washington   Los Angeles

It appears to us that further analysis is needed to decide

whether the Barber hypothesis is correct. If it can be proven that

our indications of shots do not coincide with the assassination,

then that would invalidate our findings.  But if this cannot be

proven, then it still seems necessary to further the acoustical

analysis to increase confidence in the conclusions to be derived

from the acoustical evidence.  In the following paragraphs we present

our ideas on how these further studies should be done.

A.   STUDY ORIGINALITY AND INTEGRITY OF RECORDED ACOUSTICAL EVIDENCE

1.   Phase demodulate the power hum on both recordings.

The phase of the 60 Hz power mains to which both recorders

(Dictabelt and Audograph) were connected would contain modulations

(perturbations) caused by various electrical equipment on the grid.

We recommend both channels to be analyzed for phase modulation using

a velocity tracking phase lock loop (PLL) which uses an analog phase

detector and a servo response optimized to estimate 60 Hz phase

deviations within about a 100 Hz modulation bandwidth.  The PLL

should be preceded by a very sharp bandpass filter, centered at the

power frequency (nominally 60 Hz).

2.   Compare phase modulation on the two channels as a function of time.

The Channel 2 recording is discontinuous in time, owing to the

voice-actuated recording switch.  The task is to match the phase

modulations on the two channels, so as to determine where each part

of the Channel 2 recording fits on the Channel 1 recording - if

anywhere.

3.   Study the gain discontinuity and power-hum harmonic-strength

discontinuities seen on the Channel 2 recording at

about 12:30 (nominal time of the assassination).

Look for phase discontinuities in hum here.  Look for other

indications of a splice or overlay.

4.   Look for indications of double recording on both channels, by

examining the number of hum signals that have been recorded.

This can be done by very narrow filtering or by looking for

beats in the hum waveforms.

bbn

The results of these four studies should determine whether both

recordings are single recordings or whether they have been superposed.

The correct time alignments between Channel 1 and the various segments

of the Channel 2 recording should be established also.  If the common

phrase was recorded originally and simultaneously on both channels,

and if both recordings are faithful copies of the originals, then

the Barber hypothesis is true.  And if true, the recorded sounds

that we decided were probably gunfire would not be gunfire.  But,

if not provable as true, then the truth must be found as the most

likely choice between the Barber hypothesis, our interpretation of

the acoustical evidence, or any other interpretation that may enjoy

corroboration of any kind.

The studies listed under "A" can probably show whether the

Barber hypothesis is proven or not.  If not proven we recommend

studies "B".

B.   ENHANCED ECHO PATTERN MATCHING

Owing to time and funding limitations, our original study devoted

only about three days to the process of matching the acoustical

reconstruction echo patterns with the sound patterns we had found

on the DPD recording.  This study "B" would be undertaken to add

some additional features to the matching process and to perform the

matches in a systematic way on a computer.  We suggest the use of

crosscorrelation, where both waveforms (echo patterns and DPD

recording) would be pre-processed as follows:  Each 10 ms time

window would be characterized by the amplitude and phase of the

largest peak occuring within the window.  Amplitude would be

quantized into two or three values, separated by about 10 dB.  The

best quantization would need to be studied in light of the expected

uncertainty in the directivity pattern of the motorcycle-mounted

microphone.

The matching process would amount to the continuous crosscorrelat1on

of each of the echo patterns (about 500) with 60 sec of the DPD recording

(6000 crosscorrelation values per echo pattern).  The results of

this effort would provide the data necessary to recompute the

probabilities that four shots are uniquely revealed by the DPD

recording.  In other words, if the Barber hypothesis cannot be

proved, the results of this study would increase the confidence with

which the acoustical evidence can be said to reveal gunshot times

and origins.  If the results show that gunshot sounds are probably

bbn

not on the DPD tape, then our original conclusions would be rejected.

If the results show that gunshots probably are on the tape, then

we suggest the following studies "C" to further increase confidence

in the result, and in particular, the conclusions about the third

shot.

C.   ADDITIONAL WORK OF THE WEISS AND ASCHKENASY (WA) TYPE

Owing to time and funding limitations, Weiss and Aschkenasy

applied their analytical extension of our pattern matching procedure

to the third shot only.  We suggest that their procedure be applied

to the other three shots as well.  The WA procedure will need to

be extended to include the third (vertical) dimension for the TSBD-

originated shots, because the source-reflector-receiver is not in

a horizontal plane. There is little doubt about the origin and

approximate timing of these other shots and so if their procedure

works for them too, then their procedure will be validated independently of

the controversial third shot.

We found that WA had placed the onset of the third shot echo

pattern at about 300 ms earlier than had our pattern matching

procedure.  The NAS Committee decided that this inconsistency was

an error, and therefore supported their view that either or both

of the tests were wrong.  We pointed out to the NAS Committee that

both the relevant signals on the DPD tape and the echo pattern contain

two similar bursts of echoes that are separated by about 300 ms,

and that we had simply matched on the second.  The second burst is

an echo of the first burst after its sound is reflected from the

face of the DCRB, which is nearly perpendicular to a line connecting

the Knoll with the microphone and about 150 ft from the microphone.

We suggest that the WA procedure be repeated with explicit attention

paid to this reflecting feature.

If the results of "C" show that the other three shots do not

allow an accurate position of the motorcycle microphone to be found,

or if the positions that are found are inconsistent with our knowledge

of the motorcycle trajectory, then the truth of the third shot from

the Knoll would be in question.  If the results do produce an

acceptable motorcycle trajectory, then the truth of the third shot

from the Knoll would be greatly enhanced.  In this latter event,

we suggest studies "D" that are designed to improve the credibility

of the acoustical evidence by answering questions that have arisen

about the compatibility of our interpretation of the acoustical

evidence with non-acoustical evidence.

bbn

D.   COMPATIBILITY OF ACOUSTICAL WITH NON-ACOUSTICAL EVIDENCE.

1.   Comparison of shot instants with "jiggle" analyses.

There are at least three separate analyses of the jiggle of

the Zapruder camera, presumably in reaction to the gun shots.  These

three analyses have yielded different results, so our first suggestion

is to compare them.  A quantitative analysis of camera/operator motion

induced by both muzzle blast and shock wave is needed first to enable

those jiggle features to be identified that are most likely to have

been caused by gunfire directly - as opposed to features caused by

psychomotor action.

Once a best estimate is made from the jiggle data about the

timing of each event that could be a shot, then these times would

be compared to the times found by studies "C".  These comparisons

must include tape speed corrections as well as time-of-flight

corrections.

2.   Estimate motorcycle speed

We believe that a harmonic analysis of the amplitude modulation

on the motorcycle noise can determine the motorcycle speed as a

function of time on the DPD recording (with ambiguities due to gear

position).  We suggest that this analysis be done, to show the speed

trajectory during times around 12:30.  This trajectory would establish

whether the motorcycle was moving along in the motorcade or not.

We note that if the NAS Committee were correct then the now-apparent

slowing of the motorcycle did not occur until about one minute after

the assassination.  If this analysis confirms the now-apparent slowing

of the motorcycle, then the NAS idea that the motorcycle radio passed

a Channel 2 receiver to hear the "hold everything" would be voided.

3.   Measure doppler shift on siren sounds

A few minutes after the assassination, the sounds of sirens

are heard on the Channel 1 recording.  There appears to be a doppler

shift as the sounds first intensify and then fade away.  We suggest

that the instantaneous frequency as a function of time be estimated

and plotted.  From this can be estimated the closest point of approach

and relative speeds of the motorcycle and the siren vehicle.  These

estimates will enable one to chose among the various explanations

of the origins of the siren sounds.

bbn

4.   Photographic examination of McLain's trajectory

It is important to compare our best photographic estimates of

McLain's trajectory with our best acoustical estimates - those from

"C".  We understand that further work has been done here, but not

having seen it we don't know if an adequate analysis has been done.

You have also asked us to estimate the cost of doing the studies

that we have suggested.  We offer the following figures as a guide.

It is assumed that those who do the work have the necessary equipment

and skills in hand.

A.1      1 mm (scientist) and 1 mm (technician)

A.2      1/2 mm (scientist

A.3      1 mm (scientist and l/2 mm (technician)

A.4      1/2 mm (scientist and 1/2 mm (technician)

B        5 mm (programmer) and 4 mm (scientist)

C        2 mm (scientist) and 6 mm (technician)

D.1      2 mm (scientist) and 2 mm (technician)

D.2      1 mm (scientist) and 3 mm (technician)

D.3      1 mm (scientist) and 2 mm (technician)

D.4      1/2 mm (scientist and 2 mm (technician).

As a rough estimate, the cost of a programmer or a technician would

be about $50/hour, and the cost of a scientist would be about $70/hour.

The NAS Committee report contained a list of 13 studies that

they suggested could shed additional light on the subject.  We are

in general agreement with their suggestions.  All of their suggestions

are incorporated in some way in our suggestions, except for their

Nos. 2, 6, and 10.

We hope this list of topics for further study will be useful

to you.  Although we do not wish to perform any of these studies

ourselves, we will discuss our ideas with whichever investigators

you decide upon.

Very truly yours,

signed/

Jim Barger

James E. Barger

Chief Scientist

JEB:bd

bbn